# Controller Readiness Audit - Real Controller Connection

## Context

Audit to verify the Go API server and web interface are 100% ready to connect to a real RTX controller this weekend. Cross-referenced all IPC commands and shared memory IDs from the RTX source against what the API and web interface implement.

## VERDICT: API Protocol Layer is Complete

Every command and shared memory ID needed for normal production operation is implemented. No gaps in the core protocol.

## Commands: Controller → Host (API RECEIVES)

| Cmd | Name | RTX Source | API Handles | Notes |
| --- | --- | --- | --- | --- |
| 302 | SHM_WRITE | Yes | YES | Dispatches by shmID, sends ACK_MSG back |
| 303 | DREC_RESP | Yes | YES | Drop record acknowledgement |
| 305 | BLK_DREC_RESP | Yes | YES | Block of drop records |
| 306 | ERROR_MSG | Yes | YES | Severity + message, forwarded to web |
| 307 | DEBUG_MSG | Yes | YES (logs only) | Logged, not forwarded to web |
| 308 | DREC_MSG | Yes | YES | Individual drop record, stored in DB |
| 310 | PRN_BCH_REQ | Yes | YES | Batch completion, sends PRN_BCH_RESP back |
| 320 | PRN_BCH_PRE_REQ | Yes | YES | Pre-label request, sends PRN_BCH_PRE_RSP back |
| 327 | LC_CAPTURE_INFO | Yes | YES | Waveform samples forwarded to web |

No gaps. All inbound commands handled.

## Commands: Host → Controller (API SENDS)

| Cmd | Name | RTX Source | API Sends | Trigger |
| --- | --- | --- | --- | --- |
| 309 | ACK_MSG | Yes | YES | Auto after every SHM_WRITE received |
| 311 | PRN_BCH_RESP | Yes | YES | Auto after PRN_BCH_REQ processed |
| 312 | SET_DATE_TIME | Yes | YES | On initial TCP connection |
| 317 | IND_MODE_WT_LIM | Yes | YES | Web: Exit Display Load Cell |
| 321 | PRN_BCH_PRE_RSP | Yes | YES | Auto after PRN_BCH_PRE_REQ |
| 322 | CLEAR_TOTS | Yes | YES | Web: Clear Totals |
| 324 | HOST_HEARTBEAT | Yes | YES | Every 10 seconds automatically |
| 326 | SET_LC_CAPTURE | Yes | YES | Web: Capture Load Cell run/stop |
| 302 | SET_MODE (shmID=69) | Yes | YES | Web: mode changes (RUN/RAW/TARE/SPAN) |
| 302 | SET_RAW_SCALE (shmID=70) | Yes | YES | Web: Display Load Cell scale select |
| 302 | SET_LINE_SETTINGS | Yes | YES | Web: Line Setup save |
| 302 | SET_SYNC_SETTINGS (shmID=14) | Yes | YES | Web: Sync Control save |
| 302 | SET_WEIGH_SETTINGS | Yes | YES | Web: Weigh Settings save |
| 302 | SET_GRADE_SETTINGS | Yes | YES | Web: Grade Settings save |
| 302 | SET_DROP_OFFSETS (shmID=16) | Yes | YES | Web: Drop Offsets save |
| 302 | SET_SCHEDULE (shmID=67) | Yes | YES | Web: Schedule save |
| 302 | SET_TARE_TIMES (shmID=2) | Yes | YES | Web: Tare calibration |
| 302 | SET_TARE_MODE (shmID=69) | Yes | YES | Web: Tare mode select |
| 302 | SET_SPAN_BIAS (shmID=32) | Yes | YES | Web: Auto Span |
| 302 | SET_LOADCELL_SETTINGS | Yes | YES | Web: Load cell config |
| 302 | SET_ISYS_DROPS (shmID=20) | Yes | YES | Web: InterSystems drops |
| 325 | SET_MANUAL_TARE | Yes | YES | Web: Manual tare edit |

No gaps. All outbound commands implemented.

## Commands: Defined but NOT Sent (Intentionally)

These exist in the RTX source but are NOT needed for normal operation:

| Cmd | Name | Why Not Needed |
| --- | --- | --- |
| 301 | SHM_READ | Controller pushes data via SHM_WRITE; host doesn’t need to poll |
| 304 | BLK_DREC_REQ | Drop record recovery - could be useful but not critical |
| 313 | RESTART_APP | Admin command - not in Delphi UI either |
| 314 | SHUTDOWN_NT | Admin command |
| 315 | RESTART_NT | Admin command |
| 316 | RESET_BCH_NUMS | Batch number reset - rarely used |
| 318 | AUTO_BIAS_MODE | Not exposed in UI |
| 319 | CLEAR_BATCH_RECS | Batch record cleanup |
| 323 | CLEAR_MAN_BATCH | Manual batch clear |
| 328-331 | InterSystems line-to-line | These go between controllers, not from host |

None of these are needed for this weekend’s test.

## Shared Memory IDs: Inbound (Controller sends via SHM_WRITE)

| shmID | Name | Frequency | API Parses | Web Uses |
| --- | --- | --- | --- | --- |
| 52 | DispShackle | ~200-500ms | YES | LINE_STATUS (weight, grade, drop, shackle) |
| 78 | SyncStatus | On event | YES | SYNC_STATUS (zeroed flags) |
| 80 | WeighZero | On event | YES | LINE_STATE (scale zeroed) |
| 88 | SystemOutRecords | ~1000ms | YES | LINE_STATUS + TOTALS_DATA (SPM, BPM, totals, raw weight, schedule/drop status) |
| 71 | ZeroCounter | On event | No (logged) | Not needed |
| 79 | ShackleStatus | On event | No (logged) | Not needed (DispShackle covers this) |
| 83 | GradeShackle | On event | No (logged) | Not needed |
| 84 | SysSettings | On read | No (logged) | Settings come from database |
| 85 | ScaleSettings | On read | No (logged) | Settings come from database |
| 86 | Tares | On read | No (logged) | Not used in web UI |
| 91 | MailboxStatus | On event | No (logged) | Diagnostic only |
| 92 | AppVersion | Once | No (logged) | Not displayed |
| 93 | CommVersion | Once | No (logged) | Not displayed |

The 4 critical shmIDs (52, 78, 80, 88) are fully parsed. The rest are informational/diagnostic and the API logs them without parsing. The controller does NOT require the host to process these - it just sends them.

## Shared Memory IDs: Outbound (API sends settings TO controller)

| shmID | Name | API Builder | Status |
| --- | --- | --- | --- |
| 1 | Shackles | buildSetLineSettingsMsg | YES |
| 2 | TareTimes | buildSetTareTimesMsg | YES |
| 3 | GibMin | buildSetWeighSettingsMsg | YES |
| 4 | MinBird | buildSetWeighSettingsMsg | YES |
| 5 | LastDrop | buildSetWeighSettingsMsg | YES |
| 7 | GibDrop | buildSetWeighSettingsMsg | YES |
| 8 | DropOn | buildSetLineSettingsMsg | YES |
| 9 | SyncOn | buildSetLineSettingsMsg | YES |
| 10 | SkipTrolleys | buildSetLineSettingsMsg | YES |
| 12 | MBSync | buildSetLineSettingsMsg | YES |
| 13 | MBOffset | buildSetLineSettingsMsg | YES |
| 14 | SyncSettings | buildSetSyncSettingsMsg | YES |
| 16 | DropSettings | buildSetDropOffsetsMsg | YES |
| 17 | Grading | buildSetGradeSettingsMsg | YES |
| 18 | ResetGrading | buildSetGradeSettingsMsg | YES |
| 19 | GradeArea | buildSetGradeSettingsMsg | YES |
| 20 | IsysDropSettings | buildSetIsysDropsMsg | YES |
| 23 | BatchOverrideDelay | buildSetLineSettingsMsg | YES |
| 24 | ReassignTimerMax | buildSetLineSettingsMsg | YES |
| 26 | ZeroNumber | buildSetLineSettingsMsg | YES |
| 27 | AutoBiasLimit | buildSetLineSettingsMsg | YES |
| 28 | NumAdcReads | buildSetLoadcellSettingsMsg | YES |
| 30 | OffsetAdc | buildSetLoadcellSettingsMsg | YES |
| 32 | SpanBias | buildSetSpanBiasMsg | YES |
| 33 | LoadCellType | buildSetLoadcellSettingsMsg | YES |
| 35 | LCSampleAdj | buildSetLoadcellSettingsMsg | YES |
| 67 | Schedule | buildSetScheduleMsg | YES |
| 69 | OpMode | buildSetModeMsg | YES |
| 70 | RawScale | buildSetRawScaleMsg | YES |

shmIDs NOT sent (defined but not in API): | shmID | Name | Risk | |——-|——|——| | 6 | NumDrops | LOW - Set once, controller has it in SRAM | | 11 | MissedBirdEnable | LOW - Controller has in SRAM | | 15 | Order | LOW - Drop order, controller has in SRAM | | 21 | IsysLineSettings | LOW - InterSystems line config, controller has in SRAM | | 22 | MiscFeatures | LOW - Misc flags, controller has in SRAM | | 25 | NumScales | LOW - Set once, controller has in SRAM | | 29 | AdcMode | LOW - ADC mode, controller has in SRAM |

These are settings the controller loads from its battery-backed SRAM on boot. The Delphi app only sent them when explicitly changed. Since the controller already has correct values in SRAM from previous Delphi sessions, these are NOT needed for this weekend.

## Web Interface Message Coverage

All messages the web sends are handled by the API and forwarded to the controller. Full list verified:

- SET_MODE (RUN/RAW/TARE/SPAN) ✓

- SET_RAW_SCALE ✓

- SET_IND_MODE_WT_LIM ✓

- SET_LC_CAPTURE ✓

- CLEAR_TOTALS ✓

- SET_LINE_SETTINGS ✓

- SET_SYNC_SETTINGS ✓

- SET_WEIGH_SETTINGS ✓

- SET_GRADE_SETTINGS ✓

- SET_DROP_OFFSETS ✓

- SET_SCHEDULE ✓

- SET_TARE_TIMES ✓

- SET_TARE_MODE ✓

- SET_SPAN_BIAS ✓

- SET_LOADCELL_SETTINGS ✓

- SET_ISYS_DROPS ✓

- SET_MANUAL_TARE ✓

- RESET_BATCH ✓

- CLEAR_UNASSIGNED ✓

All messages the web receives are generated by the API from controller data: - LINE_STATUS (from shmID=52 + 88) ✓ - TOTALS_DATA (from shmID=88) ✓ - DROP_RECORD (from cmd 308) ✓ - ERROR_MSG (from cmd 306) ✓ - LC_CAPTURE_INFO (from cmd 327) ✓ - SYNC_STATUS (from shmID=78) ✓ - LINE_STATE (from shmID=80) ✓ - BATCH_RECORD (from cmd 310) ✓ - BRIDGE_CONNECTED/DISCONNECTED ✓

## POTENTIAL ISSUES FOR THIS WEEKEND

### 1. TCP Interface.exe Must Be Deployed on Controller

Status: VERIFY

The API connects directly to the controller via TCP port 5000. The controller must be running the TCP version of Interface.exe (not the original named pipe version). Verify: - Is the TCP Interface.exe installed at D:\Gainco\Interface.exe on the controller? - Is it configured to listen on port 5000? - Is it running as a service?

If NOT deployed, you’ll need to: 1. Copy rtx_source/Interface/Release/interface.exe to the controller 2. Or use the Bridge approach instead (Windows PC + named pipes)

### 2. API Configuration (api.yaml)

Status: VERIFY

When connecting to a real controller, the api.yaml must have controllers enabled:

controllers:
  - line_id: 1
    host: 192.168.100.11  # Real controller IP
    port: 5000
    enabled: true          # MUST be true

Currently for simulator mode, all controllers are enabled: false. Switch this before connecting.

### 3. Database Settings vs Controller SRAM

Status: LOW RISK

The controller loads settings from battery-backed SRAM on boot. Our database has settings from simulator testing. If these differ, settings displayed in the web UI may not match what the controller is actually using.

For this weekend: the web UI displays settings from the database, and pushes changes to the controller when you save. If the controller’s SRAM settings don’t match, save each settings page once to sync them.

### 4. Weight Scaling Factor

Status: VERIFY

The API divides raw weight values by 10000.0 to get pounds. Verify this matches the real controller’s countsPerPound setting. The loadcell_config table has counts_per_pound - make sure it matches the real load cell calibration.

### 5. Struct Byte Alignment

Status: LOW RISK

The Go API uses encoding/binary.LittleEndian to parse binary structs. The RTX controller (MSVC compiler) produces little-endian x86 data. The protocol.go parsers use explicit byte offsets rather than struct mapping, which avoids alignment issues. This should just work.

### 6. Checksum Calculation

Status: IMPLEMENTED

The API calculates checksums the same way as the RTX code: sum of all bytes from byte 4 (DestLineID) onward, stored as uint32 at bytes 0-3. Verified in protocol.go BuildShmWriteMessage() and BuildCommandMessage().

## PRE-CONNECTION CHECKLIST

Before connecting to the real controller this weekend:

- ☐ Verify TCP Interface.exe is installed on controller (or plan to use Bridge)

- ☐ Update api.yaml: set controller enabled: true, correct IP and port

- ☐ Verify controller IP is reachable: ping 192.168.100.11

- ☐ Check loadcell_config.counts_per_pound matches real calibration

- ☐ Restart API after config change: sudo systemctl restart overhead-api

- ☐ Open web UI, watch for LINE_STATUS data flowing

- ☐ Verify heartbeat keeps connection alive (check API logs)

- ☐ Test one settings save (e.g., Weigh Settings) to verify outbound protocol

- ☐ Watch for ERROR_MSG from controller in web UI toasts

- ☐ If using Bridge instead of direct TCP: update database lines table with Bridge IP/port, keep controllers disabled in api.yaml